Table 1.  Materials composition and simulation transport parameter used for MC simulation in TOPAS.

| Materials | Elements & Percentages (%) |  |  | Density (g/cm3) |
| --- | --- | --- | --- | --- |
| Iridium (G4_Ir) | Ir (100 %) |  |  | 22.42 |
| AISI 316L Stainless Steel | C (0.03%), N (0.1%), Si (0.75%), P(0.045%), S (0.03%), Cr (17%), Mn (2%), Ir (65.545%), Ni (12%), Mo (2.5%) |  |  | 7.8 |
| AISI 304 Stainless Steel | C (0.03%), N (0.1%), Si (1%), P (0.045%), S (0.015%), Cr (18%), Mn (2%), Ir (69.81%), Ni (9%) |  |  | 5.6 |
| G4_Air /Dry Air | C (0.0124%), N (75.5268%), O (23.1781%), Ar (1.2827%) |  |  | 1.20479 x 10-3 |
| G4_Water | H (11.1894%), O (88.8106%) |  |  | 1 |
| TOPAS Monte Carlo simulation parameters |  |  |  |  |
| Parameters |  | Description | Controls |  |
| EMRangeMin |  | 10 eV | Minimum track range for particle transport; below this energy deposited locally |  |
| EMRangeMax |  | 500 MeV | Maximum EM track range |  |
| CutForAllParticles (γ; e-; e+) |  | 0.05 mm | Range cut for secondary production |  |
| SetProductionCutLowerEdge |  | 100 eV | Lower threshold for secondary particle production |  |
| SetProductionCutHighEdge |  | 30 MeV | Upper threshold for secondary particle produced ignoring cutoff |  |
| Voxel size |  | 1 mm3 | Imported 3D calculation dose grid of cubic voxels. |  |
| Particle histories |  | ≈ 109 |  |  |

Table 2.  Materials, mass densities, and elemental composition of TG-186 generic applicator modeled in TOPAS for MC simulation.

| Components | Materials | Elements & Percentages (%) | Density (g/cm3) |
| --- | --- | --- | --- |
| Cylinder Body & Cap | PMMA (Lucite) | H (8.0538 %), C (59.9848%), O (31.9614%) | 1.19 |
| Insertion tube | AISI 316L Stainless Steel | P (0.045%), C (0.08%), Si (1 %), Mn (2%), Ni (9.5%), Cr (19%), Ir (68.375%), | 8 |
| Shield | DensimetD176 | Ir (2.5%), Ni (5%), W (92.5%) | 17.6 |
| Air40 | 40% Humidity Air | C (0.0123%), H (0.0732%), Ar (1.2743%) O (23.6077%), N (75.0325%) | 1.19 |